# KIC Fracture Toughness Test Report

ASTM E399 - Standard Test Method for Linear-Elastic

Plane-Strain Fracture Toughness of Metallic Materials

Report Number: {{report_number}}

# 1. Test Information

| Test Project: | {{test_project}} |
| --- | --- |
| Customer: | {{customer}} |
| Specimen ID: | {{specimen_id}} |
| Material: | {{material}} |
| Test Date: | {{test_date}} |
| Test Temperature: | {{temperature}} °C |
| Test Standard: | ASTM E399 |

# 2. Specimen Dimensions

| Parameter | Value | Unit |
| --- | --- | --- |
| Specimen Type | {{specimen_type}} | - |
| Width W | {{W}} | mm |
| Thickness B | {{B}} | mm |
| Net Thickness B_n | {{B_n}} | mm |
| Crack Length a_0 | {{a_0}} | mm |
| Span S (SE(B) only) | {{S}} | mm |

# 3. Material Properties

| Parameter | Value | Unit |
| --- | --- | --- |
| Yield Strength σ_ys | {{yield_strength}} | MPa |
| Young's Modulus E | {{youngs_modulus}} | GPa |
| Poisson's Ratio ν | {{poissons_ratio}} | - |

# 4. Test Results

| Parameter | Value | U (k=2) | Requirement | Unit |
| --- | --- | --- | --- | --- |
| Maximum Force P_max | {{P_max}} | ±{{P_max_uncertainty}} | {{P_max_req}} | kN |
| Conditional Force P_Q | {{P_Q}} | ±{{P_Q_uncertainty}} | {{P_Q_req}} | kN |
| P_max/P_Q Ratio | {{P_ratio}} | - | {{P_ratio_req}} | - |
| Conditional K_Q | {{K_Q}} | ±{{K_Q_uncertainty}} | {{K_Q_req}} | MPa√m |
| Fracture Toughness K_IC | {{K_IC}} | ±{{K_IC_uncertainty}} | {{K_IC_req}} | MPa√m |
| Initial Compliance | {{compliance}} | - | - | mm/kN |
| Validity Status | {{is_valid}} | - | - | - |

# 5. Force vs Displacement

{{chart}}

# 6. Validity Assessment per ASTM E399

Overall Validity: {{is_valid}}

Validity Checks:

{{validity_notes}}

# 7. Notes

The 5% secant offset method was used to determine P_Q per ASTM E399.

Validity requirements per ASTM E399:

- a/W ratio must be between 0.45 and 0.55

- B ≥ 2.5(K_Q/σ_ys)² for plane-strain conditions

- a ≥ 2.5(K_Q/σ_ys)²

- (W-a) ≥ 2.5(K_Q/σ_ys)²

- P_max/P_Q ≤ 1.10

# 8. Approval

| Role | Name | Date/Signature |
| --- | --- | --- |
| Tested by: |  |  |
| Reviewed by: |  |  |
| Approved by: |  |  |